Rhodeus BIOS update procedure:

The latest BIOS version of the board is T21 has to be loaded (A5362T21.BIN)

Procedure:

- Connect VGA, KB/M cables to the mating connectors on the board , VGA1 and KBM1  .

- Plug a Floppy disk drive to FDD1 connector on the board

- Use an ATX power supply , wire jumper the green and black wires on 2x10 main power connector and use one of the four pin FDD power connectors of ATX power supply to power up the board (PWR1) and another one to power up a floppy disk drive.

- Make a DOS bootable floppy disk

- Copy awdflash.exe and a5362t21.bin into it.

- Create an Autoexec.bat containing : awdflash a5362t21.bin /py/sn/f/r/ld .

- In this way when the system powers up, it automatically upgrades BIOS.

Diamond Systems

9/28/09