BIOS Update Procedure

 Copyright 2012

Diamond Systems Corporation

www.diamondsystems.com

General Helios Bios Update Procedure

Rev 1 August 1, 2012

# GENERAL DESCRIPTION

This document provides general instructions to upgrade the BIOS for a Helios board

# EQuipment required

- MS-DOS 6.22 Flash disk with a program named SPIFLASH is used to program the BIOS

- PS/2 Keyboard.

- PS-5V-04 power supply

- VGA Monitor

# files

- SPIFLASH.EXE – bios update tool

- 123456.ROM – the updated rom bios file

# procedure

- Prepare a DOS bootable flash disk or USB disk and copy the following files into it on C:\ of the flash disk:

SPIFLASH.exe

123456.ROM

- Connect the Helios board under upgrade on the Helios test station with the flash disk prepared above set as a boot device.

- Power up the Helios board and boot to the DOS prompt.

- To update the bios on the board, type the following at the command prompt:

  - spiflash u 123456.rom NOMD5

- Programming process should begin and ADDR should increment.

- Watch for the program to exit.

- On restart, press ‘Del’ key to go to BIOS settings.

- Go to the Exit menu and press ‘Load Setup Defaults’. Save the settings and exit the BIOS.

- The BIOS upgrade is complete now.