Micrel KSZ8842M WINCE Driver Installation Guide

Version 0.1

09/05/2007

Overview:

This document is writing for the version 1.09 or later.

The driver is developed on Micrel Load Board and has been tested for WINCE500 and WINCE600.

The document takes the KS8842 for example. For the KS8841 user, you need to change the source file TARGETNAME=KS8841 and copy the KS8842.reg to platform.reg.

- How to add driver:

Unzip the KS8842M_driver_x.xx.zip to your driver working directory.

For WINCE500:

- From the Platform menu, choose BSP Wizard and then choose Next.

- Select Modify an existing BSP, select your BSP to add a driver to, and then choose Next.

- In the BSP Customization dialog box, select Add driver to add a new device driver to the BSP definition.

- In the Catalog Item Properties dialog box, enter ‘KS8842M’ for the driver name.

- Choose the Type tab; then from the Type list, select the ‘Network’ as the driver type.

- In the Source code directory field, enter the path to the directory that contains the source code for KSZ8842XM driver.

- In the Driver resolver module field, enter the ‘KS8842.dll’.

- Choose OK to close the Catalog Item Properties dialog box and return to the Customization dialog box

- When you finish customizing your BSP, choose Next

- To close the BSP Wizard and save your changes, choose Next.

You have now added the device driver to the BSP, and the BSP or driver is displayed in the Catalog.

For WINCE600:

- From Visual Studio 2005 menu chooses File and then chooses Open. Open your WINCE600 Catalog File (xxxx.pbcxml).

- Right click the icon under Device Drivers\Networking\LocalArea Networking (LAN)devices\ and choose Add Catalog Item.

- From right pane, choose Supported CPUs field and click the right side combo box button. From the pop-up Supported CPUs dialog box, select the ARMV4I check box.

- Change the Title field text to KS8842M.

- Change Description field text to Micrel KS8842X WINCE driver.

- Choose the Modules field and click the right side combo box button.

Add KS8842.dll into the pop-up dialog box.

- Choose the Additional Variables field and click the right side combo box button. Add BSP_NIC_KS8842 into the pop-up dialog box.

- Change Unique ID field text to Micrel:KS8842M .

- From left pane, right click the KS8842M item and choose Add Source Code Link

- From right pane, choose the Path field and click right side combo box button. Then from the pop-up Browse for Folder dialog box, choose the directory included the driver source code.

- On the same pane, Change the Title field text to KS8842M.

- Save the Catalog file.

- Add driver module:

Add following code into Platform.bib file.

For WINCE500:

IF BSP_NIC_KS8842

KS8842.dll $(_FLATRELEASEDIR)\KS8842.dll      NK  SH

ENDIF

For WINCE600:

IF BSP_NIC_KS8842

KS8842.dll $(_FLATRELEASEDIR)\KS8842.dll      NK  SHK

ENDIF

- Add driver registry settings

Copy KS8842.reg file to platform.reg file

Note: you must change the IoBaseAddress and InterruptNumber under the [HKEY_LOCAL_MACHINE\Comm\KS88421\Parms] according to your board design.

- Configuration:

Because the KS8842 driver is developed on Micrel LOAD Board hardware, you need to change some configuration code according to your hardware design.

1. Remove the following code from target.h file

/* CONFIG_KS884X_LOAD_BOARD */

#undef ADDR_SHIFT

#undef ADDR_BASE

#define ADDR_SHIFT  1  // for Load Board, KS8842 address is shifted by 1, if you hardware does not do address shift, define the ADD_SHIFT 0

#define ADDR_BASE   ( 0x300 << ADDR_SHIFT )

Above code is only for Micrel Load Board.

- Change the Bus width definition according to you hardware design.

Default Bus width defined in the target.h is 16 bit.

#define SH_16BIT_DATABUS.

If you are using 32 bit data bus.  Comment this line and uncomment above line

#define SH_32BIT_DATABUS

- Enable VLAN support.

To enable the VLAN support, adding the -DVLAN_ENABLE compilation flag to source file and adding the variable VLAN_ENALE =1 to your project batch file or to the KS8842X driver. Which will let the VLAN WINCE registry settings take effect. For how to set a tagged VLAN correctly, please see the ‘How to set a tagged VALN.doc’ file.

- Enable the MIB Count

To enable the MIB count support, adding the -DMIBCOUNT_ENABLE compilation flag to source file and adding variable MIBCOUNT_ENABLE=1 to your project batch file or to the KS8842X driver.

After the MIB count support is being enabled, application can read the MIB values from WINCE registry.

Contact Information:

Charles Li

Email: charles.li@micral.com

David Cai

Email: boliang.cai@micral.com